www.hf-radio.com.au ……Phone 07 3118 5265

Barrett SB250 V5.20 Marine  Channel Profile.

Selcall & Telcall are enabled on all Transmit channels.

Emergency tone Button is enabled on Distress Frequencies.

| Channel | Transmit (kHz) | Receive (kHz) | Channel use / Comment |
| --- | --- | --- | --- |
| 0 | 2182 | 2182 | Marine Distress |
| 1 | 4125 | 4125 | Marine Distress |
| 2 | 5680 | 5680 | Marine Distress |
| 3 | 6215 | 6215 | Marine Distress |
| 4 | 8291 | 8291 | Marine Distress |
| 5 | 12290 | 12290 | Marine Distress |
| 6 | 16420 | 16420 | Marine Distress |
| 11 | 2032 | 2032 | General Marine |
| 12 | 2065 | 2065 | General Marine |
| 13 | 2079 | 2079 | General Marine |
| 14 | 2112 | 2112 | General Marine |
| 15 | 2284 | 2284 | General Marine |
| 16 | 2524 | 2524 | General Marine |
| 17 | 2638 | 2638 | General Marine |
| 18 | 3023 | 3023 | General Marine |
| 19 | 4146 | 4146 | General Marine |
| 20 | 4149 | 4149 | General Marine |
| 21 | 4483 | 4483 | General Marine |
| 22 | 4535 | 4535 | General Marine |
| 23 | 4620 | 6224 | General Marine |
| 24 | 6224 | 6224 | General Marine |
| 25 | 6227 | 6227 | General Marine |
| 26 | 6230 | 6230 | General Marine |
| 27 | 8174 | 8174 | General Marine |
| 28 | 8294 | 8294 | General Marine |
| 29 | 12353 | 12353 | General Marine |
| 30 | 12359 | 12359 | General Marine |
| 31 | 16420 | 16420 | General Marine |
| 32 | 16534 | 16534 | General Marine |

Barrett SB250 V5.10 Marine Mode continued.

| Channel | Transmit (kHz) | Receive (kHz) | Channel use / Comment |
| --- | --- | --- | --- |
| 36 |  | 4149 | Weather Rx / VMW |
| 37 |  | 6230 | Weather Rx / VMW |
| 38 |  | 8113 | Weather Rx / VMW |
| 39 |  | 12362 | Weather Rx / VMW |
| 40 |  | 16528 | Weather Rx / VMW |
| 41 |  | 4426 | Weather Rx / VMC |
| 42 |  | 6507 | Weather Rx / VMC |
| 43 |  | 8176 | Weather Rx / VMC |
| 44 |  | 12365 | Weather Rx / VMC |
| 45 |  | 16546 | Weather Rx / VMC |
| 46 |  | 6080 | ABC-Receive |
| 47 |  | 7240 | ABC-Receive |
| 48 |  | 9660 | ABC-Receive |
| 49 |  | 13360 | ABC-Receive |
| 50 |  | 15240 | ABC-Receive |
| 51 | 5455 | 5455 | VKS737 / HF-Tel |
| 52 | 8022 | 8022 | VKS737 / HF-Tel |
| 53 | 11612 | 11612 | VKS737 / HF-Tel |
| 54 | 14977 | 14977 | VKS737 / HF-Tel |
| 55 | 3995 | 3995 | VKS737 / HF-Tel |
| 56 | 6796 | 6796 | VKS737 / HF-Tel |
| 57 | 10180 | 10180 | VKS737 / HF-Tel |